| School/Department/Unit: Chemistry and Chemical Engineering | Work Activity: Gas Chromatography Analysis |  |
| --- | --- | --- |
| Assessment Completed by: DP / JO’C | Date Completed: August 2022 | Review Before: August 2024 |

NOTE: Before reading this you MUST read the ‘SOP - Energy and environmental impacts under normal, abnormal and emergency conditions’ which is Mills group web site, https://www.profandrewmills.com/leaf-documents/.  This addresses general energy and environmental impacts under normal, abnormal and emergency conditions considerations which you NEED to be cognisant of before conducting any experiment.  If you identify anything in an SOP which can be improved, please contact the LO and PI to discuss the proposed change(s) before putting them into effect.

| Activity (task step) | Hazards | Who might be harmed | Existing Control Measures (What are you already doing?) | Severity (1-5) | Likelihood (1-4) | Risk Level | What further action is necessary | Severity (1-5) | Likelihood(1-4) | Final risk level | Action by whom & by when |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Moving around the Laboratory | Slips and trips | Staff, Students, Researchers, Visitors and Cleaners | - Good housekeeping: i.e. keeping the Laboratory tidy - Elimination of tripping hazards e.g. cables are tidy - Floor in good condition - Good Lighting | 1 | 2 | L | Regular checks that the Laboratory environment is tidy, and the floor is free from obstructions and cabling is well secured | 1 | 2 | L | Academic Supervisor, All staff, daily |
| Working at a Computer Workstation linked to GC | Eyestrain, repetitive strain injuries, back and muscle pain, tension and headache | Staff, Students, Researchers | - Online training for correct use of Computer workstation - DSE Assessment completed - Assessment of DSE workstation by School DSE Assessor - Vision screening - Provision of display screen equipment aids (anti-glare screens, mouse mat, wrist rest, document holder and footrest – where required) - Rest periods, exercises and variance in work activities - Workstation and equipment set to ensure good posture and to avoid glare from lights and windows - Relevant desk of suitable height and accompanying chair / stool of suitable height - Referral to Occupational Health if required | 2 | 2 | L | Check that identified action from DSE assessment followed up. Staff to inform Line manager of any pain they have that may be linked to computer use. | 2 | 1 | L | DSE assessor, Staff |

| Activity (task step) | Hazards | Who might be harmed | Existing Control Measures (What are you already doing?) | Severity (1-5) | Likelihood (1-4) | Risk Level | What further action is necessary | Severity (1-5) | Likelihood(1-4) | Final risk level | Action by whom & by when |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Use of electrical equipment | Electric shock, burns | Staff, Students, Researchers | - Current in date PAT test label - Staff / Students / Researchers to report any defective plugs, discoloured sockets, damaged cable and on / off switches through planon or to relevant BLO (Building Liaison Officer) - Staff to put defective items out of use - Portable Appliance Testing (PAT) on annual or bi-annual basis by authorised and trained persons | 1 | 2 | L | Regular visual checks on electrical items Annual or Bi-Annual PAT | 1 | 1 | L | Staff, Academic Supervisors and Safety Co-ordinators |
| Fire in Laboratory and surrounding area | Smoke inhalation / burns | Staff, Students, Researchers and Visitors | - Fire Risk Assessment done - Evacuation plan in place - Annual Fire Drill (minimum but usually more as False Alarm also counts as a drill) - Weekly Fire Alarm checks - Fire extinguishers in place - Corridors, exits and Fire doors kept clear of obstructions - FMS (Fire Management System) Checks at weekly and monthly intervals by authorised CCE staff - Annual Fire Safety Training | 4 | 2 | H | Annual Fire Safety Training and Continual review of Fire Safety | 4 | 2 | H | Staff, Safety Co-ordinators, Academic Supervisor |
| Accidents | Injury to person requiring attention | Staff, Students, Researchers and Visitors | - Trained First Aiders - Posters in convenient locations showing where First Aiders can be contacted - First Aid boxes in area - Defibrillator available in 2 locations within building - Staff trained to know how to contact appropriate help e.g. 2222 / 5099 -Eyewash and Safety Shower in place | 2 | 2 | L | Regular review of First Aid provision and Monthly checking of Safety Eyewash | 2 | 2 | L | Safety Co-ordinators |
| Working with Glass and / or Glass sample vials | Glassware – risk of cuts from breakages / defective glassware Exposure to Contents contained in Sample vials | Staff, Students, Researchers and Visitors | - Handle Sample vials with care and Visual Inspection of vials for defects and discard those that fail inspection - Sample vials capped to prevent accidental spillage - Each student / researcher / member of staff is responsible for doing the necessary associated COSHH assessment of the material they are working with and using the necessary PPE. - Spillages and breakages to be cleaned as specified in Safety Data sheets - Glass waste and used Sample vials disposed of in suitable Glass bin - Suitable storage arrangements - PPE to be worn (Gloves, Eye Protection and Laboratory coat) - Safety Data sheets to be consulted when necessary - Good hygiene to be employed – washing of hands and no eating or drinking in area | 3 | 2 | M | Regular Inspection by Safety Co-ordinators | 3 | 1 | L | Safety Co-ordinators |
| Accidental release of significant volumes of Gas into Laboratory (Nitrogen, Compressed Air, Hydrogen) | Asphyxiation | Staff, Students, Researchers, Visitors and Cleaners | - Gas Sensors installed in Laboratory with Alarm system - Gas Sensors calibrated 6 monthly - Emergency Procedure in place for Alarm activation - Trained First Aiders - Posters in convenient locations showing where First Aiders can be contacted - First Aid boxes in area - Staff trained to know how to contact appropriate help e.g. 2222 / 5099 - Trained Breathing Apparatus users if required and notice in area - Breathing apparatus checked regularly and yearly Refresher training undertaken by users from Fire Brigade | 4 | 2 | H | Regular checking of Gas Sensors and Breathing Apparatus | 4 | 2 | H | Safety Co-ordinators |
| Gas Cylinder Use and moving | Cylinder falling (foot, toe or other limb injury), Strain to back / limbs, Hand injury when moving Cylinders, Uneven / Slippery Surfaces Unsecured / Falling Cylinder | Staff, Students, Researchers, Visitors, Contractors and members of Public | - Staff trained in Cylinder handling - Staff trained in on-line Gas Safety courses - Gloves, Safety Footwear and Eye Protection to be worn when moving Cylinders - Inspection and reporting of ground defects to Estates / BLO and Good housekeeping of area - All cylinders transported in upright position - No movement of Gas cylinders in extreme weather conditions such as snow or ice - Cylinders always secured either on trolley or manually and never left unattended - In Laboratory Cylinders always secured / Leak tests performed Regularly - Regulators checked annually for defects / Out of date Regulators red-tagged and subsequently replaced | 4 | 2 | H | Refresher Training on Gas Cylinder handling and periodic review of ground where handling | 4 | 2 | H | Safety Co- ordinators, Staff |
| Using PerkinElmer Claus 590 GC | Electrical / Use of Gases / High Temperatures | Staff, Students, Researchers and Visitors | - Current in date PAT test label - Electrical (covered previously) - Pre-check of Machine for any irregularities visible - Correct following of SOP - PPE to be worn (Gloves, Eye Protection and Laboratory coat) - Exposure to Gases covered previously - Emissions of Heat limited due to machine casing - Proper Exhaust Ventilation | 2 | 2 | L | None | 2 | 2 | L |  |
| Uptake of sample being analysed with syringe | Piercing of skin / body parts with syringe needle and possible injection of a potentially hazardous substance | Staff, Students | - Only trained users are permitted to use the instrument. - Wear appropriate PPE - Always place the syringe needle back in the case when not being used. - Handle the syringe and sample uptake as demonstrated on training by the staff member as to avoid accidents. | 2 | 2 | L | No further action necessary | 2 | 2 | L | Staff, Students |

| NAME | SIGNATURE | DATE |
| --- | --- | --- |
| DAVID PARKER |  |  |
| JACKIE O’CONNOR |  |  |