| School/Department/Unit: Chemistry & Chemical Engineering | Work activity: NOx removal measurement |  |
| --- | --- | --- |
| Assessment completed by: DH | Date completed: 5-2016 | Review Period: |

NOTE: Before reading this you MUST read the ‘SOP - Energy and environmental impacts under normal, abnormal and emergency conditions’ which is Mills group web site, https://www.profandrewmills.com/leaf-documents/.  This addresses general energy and environmental impacts under normal, abnormal and emergency conditions considerations which you NEED to be cognisant of before conducting any experiment.  If you identify anything in an SOP which can be improved, please contact the LO and PI to discuss the proposed change(s) before putting them into effect.

| Hazard/risk description | Persons exposed (who might be harmed) | Existing Control Measures (what are you already doing) | Severity (1-4) | Likelihood (1-4) | Risk Level (L,M,H, VH) | Additional controls required to achieve an acceptable level of risk | Final risk rating | Action by whom & by when |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Compressed gas; may explode if heated | Staff and students. | Gas cylinders kept away from sources of heat and ignition. Containers not exposed to temperatures exceeding 50oC. | 4 | 1 | M |  |  |  |
| Oxygen depletion / Asphyxiation | Staff and students. | VOC gas cylinder is always turned on only when the whole system is closed and exhausted out of the window. VOC gas is only turned on after the air which is to use to dilute the VOC. | 4 | 1 | M |  |  |  |
| UV light exposure | Staff and students | PPE – safety specs and gloves are used when dealing with UV light. UV lamps are in closed vented containers and can be operated from the outside. | 1 | 3 | M |  |  |  |
| Manual Handling | Staff and students | Applicable when handling gas cylinders. Cylinder trolley to be used is in the room. Wear non-slip safety shoes. | 2 | 1 | L |  |  |  |
| Electrocution | Staff and students | All equipment PAT tested annually and checked prior to use. | 3 | 1 | M |  |  |  |
| Misuse of System Operation | Staff and students | Training required to use system | 3 | 1 | M |  |  |  |

| Severity | Likelihood |  |  |  |  | Risk Rating |  | Action |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unlikely | Possible | Likely | Very likely |  | Score | Risk Level |  |  |
| Very Minor | 1 | 2 | 3 | 4 |  | 1-2 | Low |  | No further action necessary |
| Minor | 2 | 4 | 6 | 8 |  | 3-6 | Medium |  | Although tolerable – monitor activity |
| Significant | 3 | 6 | 9 | 12 |  | 8-9 | High |  | Further measures required |
| Major | 4 | 8 | 12 | 16 |  | 12-16 | Very High |  | Immediate action required – cease activity |