| School/Department/Unit: Chemistry & Chemical Engineering | Work activity: Sputter coating with Quorum Q150T |  |
| --- | --- | --- |
| Assessment completed by: AMcN | Date completed: 3-2019 | Review Period: |

NOTE: Before reading this you MUST read the ‘SOP - Energy and environmental impacts under normal, abnormal and emergency conditions’ which is Mills group web site, https://www.profandrewmills.com/leaf-documents/.  This addresses general energy and environmental impacts under normal, abnormal and emergency conditions considerations which you NEED to be cognisant of before conducting any experiment.  If you identify anything in an SOP which can be improved, please contact the LO and PI to discuss the proposed change(s) before putting them into effect.

| Hazard/risk description | Persons exposed (who might be harmed) | Existing Control Measures (what are you already doing) | Severity (1-4) | Likelihood (1-4) | Risk Level (L,M,H, VH) | Additional controls required to achieve an acceptable level of risk | Final risk rating | Action by whom & by when |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Electrocution | Staff and students | All equipment PAT tested annually and checked prior to use. | 3 | 1 | M |  |  |  |

| Severity | Likelihood |  |  |  |  | Risk Rating |  | Action |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unlikely | Possible | Likely | Very likely |  | Score | Risk Level |  |  |
| Very Minor | 1 | 2 | 3 | 4 |  | 1-2 | Low |  | No further action necessary |
| Minor | 2 | 4 | 6 | 8 |  | 3-6 | Medium |  | Although tolerable – monitor activity |
| Significant | 3 | 6 | 9 | 12 |  | 8-9 | High |  | Further measures required |
| Major | 4 | 8 | 12 | 16 |  | 12-16 | Very High |  | Immediate action required – cease activity |