| School/Department/Unit: Chemistry & Chemical Engineering | Work activity: Water and Chloride Oxidation using Photoanodes |  |
| --- | --- | --- |
| Assessment completed by: Chris O’Rourke | Date completed: 1-2019 | Review Period: 2 years |

NOTE: Before reading this you MUST read the ‘SOP - Energy and environmental impacts under normal, abnormal and emergency conditions’ which is Mills group web site, https://www.profandrewmills.com/leaf-documents/.  This addresses general energy and environmental impacts under normal, abnormal and emergency conditions considerations which you NEED to be cognisant of before conducting any experiment.  If you identify anything in an SOP which can be improved, please contact the LO and PI to discuss the proposed change(s) before putting them into effect.

| Hazard/risk description | Persons exposed (who might be harmed) | Existing Control Measures (what are you already doing) | Severity (1-4) | Likelihood (1-4) | Risk Level (L,M,H, VH) | Additional controls required to achieve an acceptable level of risk | Final risk rating | Action by whom & by when |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| UV light exposure | Staff and students | Eye protection and gloves are worn at all times. During all irradiations, the light sources are completely covered/enclosed to minimise exposure. | 1 | 2 | L |  |  |  |
| Electrocution | Staff and students | All equipment PAT tested annually and checked prior to use. | 3 | 1 | M |  |  |  |
| Exposure to sharps | Staff and students | The syringe needle at the end of the tubing from the argon regulator is inserted into the PTFE tubing of the EChem cell using pliers to avoid possible injury. | 1 | 2 | L |  |  |  |

| Severity | Likelihood |  |  |  |  | Risk Rating |  | Action |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Unlikely | Possible | Likely | Very likely |  | Score | Risk Level |  |  |
| Very Minor | 1 | 2 | 3 | 4 |  | 1-2 | Low |  | No further action necessary |
| Minor | 2 | 4 | 6 | 8 |  | 3-6 | Medium |  | Although tolerable – monitor activity |
| Significant | 3 | 6 | 9 | 12 |  | 8-9 | High |  | Further measures required |
| Major | 4 | 8 | 12 | 16 |  | 12-16 | Very High |  | Immediate action required – cease activity |